# Highlights

# Optimizing warnings from hydrologic ensemble prediction systems for improved decision making

Jesu´s  Casado-Rodr´ıguez,  Peter  Salamon,  Stefania  Grimaldi,  Corentin  Car- ton De Wiart, Christel Prudhomme, Calum Baugh, Ervin Zsoter, Ilias Pech- livanidis, Nina Bosshard, Michaela Mikulickova

- Probabilistic NWP models outperform deterministic counterparts in terms of flood warning skill.

- Using persistence (consecutive forecasts) as a warning criterion is not  effective in probabilistic systems.

- Building grand ensembles adds value to the skill of the best performing NWP but this added value decreases with lead time.

# Optimizing warnings from hydrologic ensemble prediction systems for improved decision making

Jesu´s Casado-Rodr´ıgueza, Peter Salamona, Stefania Grimaldia, Corentin Carton De Wiartb, Christel Prudhommeb, Calum Baughb, Ervin Zsoterb,

Ilias Pechlivanidisc, Nina Bosshardc, Michaela Mikulickovad

aEuropean Commission, Joint Research Centre, Directorate E - Space, Security and Migration, Via Enrico Fermi 2749, Ispra, 21027, (VA), Italy

bEuropean Centre for Medium Range Weather Forecasts, Shinfield Park, Reading, WC2R 2LS, UK

cSwedish  Meteorological  and  Hydrological  Institute,  Folkborgsv¨agen 17,  Norrk¨oping,  Sweden

dSlovak  Hydrometeorological  Institute,  Jes´eniova  17,  Bratislava,  Slovakia

# Abstract

Early Warning Systems are critical in mitigating the impacts of floods, the most frequent and economically damaging natural hazard worldwide. These systems rely on hydrological simulations driven by meteorological forecasts, both of which are inherently uncertain. Ensemble Prediction Systems (EPS) were developed decades ago to account for these uncertainties by generating multiple future scenarios that portray the forecast uncertainty. The advan- tages of probabilistic modelling have been proven over the last decades both in the fields of meteorology and hydrology. In order to issue a flood warning, the probabilities provided by EPS need to be converted into dichotomous values of ”warning” or ”not warning”. This conversion is often done by ap- plication of thresholds, whose definition is pivotal for the final skill of the system. While meteorological and hydrological models are under continuous development, the methodologies for converting discharge forecasts into flood warnings lack continuous evaluation. In this paper, we use discharge simulations from the European Flood Awareness System (EFAS) to assess the skill of the four Numerical Weather Prediction (NWP) models of different nature —probabilistic and deterministic—within the system, and search for methods to combine all the NWP into a grand ensemble that enhances overall skill. Additionally, we evaluate the effects of the current notification criteria —probability threshold

Preprint submitted to Journal of Hydrology	February 1, 2024

and persistence— and determine their optimal values. Our results demon- strate that probabilistic NWP models outperform deterministic ones in terms of flood warning skill, which demonstrate skill only at short lead times (up to 3 days). Additionally, our findings sug- gest that the persistence criterion is beneficial for removing inconsistencies between consecutive forecasts in deterministic NWP, but should be omitted from probabilistic systems. By applying optimal notification criteria to a single probabilistic NWP (ECMWF-ENS), we observe an improvement of up to 6.4% in skill compared with the current EFAS procedure, which employs four NWP models. When compared with the optimized ECMWF-ENS warnings, grand ensembles yield significant improvements in skill (up to 4.2%) only within the first two lead days. Two methods to build the grand ensem- ble stand out —equiprobability among all model runs and skill weighted, and we discuss advantages and drawbacks of these two approaches. Overall, our study underscores the importance of selecting appropriate flood notification criteria and limits the value added to the grand ensemble by deterministic NWPs to the shortest lead time range.

Keywords: EFAS, early warning system, flood warnings, probabilistic forecasting

1     1.  Introduction

2	Floods are the natural disaster that causes the biggest economical losses in

3     Europe WMO (2021). In the European Union (EU), flood damages amounted

4     to a total cost of approximately EUR 280 billion in the period 1980-2022,

5     representing 43% of all meteorological hazards EEA (2023).  The historical

6     data show an increasing tendency in flood damages over Europe EEA (2023);

7     Paprotny et al. (2018), and climate projections indicate an exacerbation of

8     this trend with increasing frequency and severity of extreme events Alfieri

9     et al. (2017); Martina Angela Caretta et al. (2023).  In this scenario, Flood

10     Early Warning Systems (FEWS) emerge as one of the multiple adaptation

11     measures to be taken.    The major flood event in the Elbe River in 2002

12     triggered the development of the European Flood Awareness system (EFAS),

13     a continental scale FEWS that has been operational since 2012 as part of the

14     Copernicus Emergency Management Service and whose monetary value has

15     been proven Pappenberger et al. (2015).   The magnitude of the flood that

16     hit Germany and Belgium in July 2021, the most costly in the last 40 years,

17     EEA (2023) was a reminder of the importance of FEWS and their need for

18     continuous improvement. On a global scale, the United Nations launched the

19     initiative Early Warnings for All to ensure that the entire world

20     is warned about weather, water and climate hazards by 2027 Egerton et al.

21     (2023).

22	FEWS rely on meteorological forecasts generated by Numerical Weather

23     Prediction (NWP) models.   Over the past thirty years,  Ensemble Predic-

24     tion Systems (EPS) have been developed within the realm of NWP models

25     to address uncertainties in the initial conditions.	EPS  provide  equiprob-

26     able forecasts at any given time and location, enabling the quantification

27     and dissemination of probabilistic forecasts Cloke and Pappenberger (2009).

28     In comparison to deterministic models, EPS have demonstrated enhanced

29     consistency across consecutive forecasts Buizza (2008) and improved precip-

30     itation forecasts at longer lead times Molteni et al. (1996).

31	Approximately two decades ago, the success of EPS in meteorology led

32     to the adoption of this paradigm in flood forecasting Diomede et al. (2008);

33     Bartholmes et al. (2009); Thielen et al. (2009); Yang et al. (2020).  Hydro-

34     logical Ensemble Predictions Systems (HEPS) propagate the uncertainties in

35     the meteorological forecasts by forcing hydrological models with the members

36     of a meteorological ensemble. Although this approach is the most common,

37     it only considers the uncertainty in the initial meteorological condition, but

38     does not include other sources of uncertainty, such as model structure

39     or model parameterization Cloke and Pappenberger (2009).

40	The decision to issue weather-related warnings, such as floods, is delicate

41     as it might result in significant costs, either due to unpreparedness if an event

42     is missed or the mobilization of resources in false alarms. Weather forecast-

43     ing systems often issue a higher number of false alarms than missed events

44     Bouttier and Marchal (2023), despite the potential risk of eroding trust in

45     the system, known as the ’cry wolf’ effect LeClerc and Joslyn (2015).  This

46     latter research showed that an increase in false alarms diminishes the credi-

47     bility of the system, leading end-users to disregard precautionary measures.

48     However, they also demonstrated that incorporating uncertainty estimates in

49     warning messages can assist users in making informed decisions. Therefore,

50     HEPS have the potential to enhance both the skill and trustworthiness of

51     flood warning system, if particular attention is given to the balance between

52     false alarms and missed events.

53	EFAS is the hydrological forecast component of the Copernicus Emer-

54     gency Management Services (CEMS) of the European Commission Thielen

55     et al. (2009). EFAS is an operational system that monitors and forecasts

56     floods in an extensive European domain, encompassing not only European

57     Union member states. Its primary objective is to enable proactive measures

58     in anticipation of significant flood events. EFAS functions as a complemen-

59     tary system to national or regional counterparts, with a specific focus on

60     trans-boundary rivers and medium-range forecasts.

61	With the release of version 4.0,  EFAS has been generating forecasts

62     twice a day, with a 6-hour resolution and lead times extending up to 10

63     days.	It couples four weather forecasts of diverse nature —deterministic

64     and probabilistic— with the distributed, processed-based hydrological model

65     LISFLOOD-OS Burek et al. (2013).  The simulation produces a set of wa-

66     ter fluxes and state variables, from which river discharge is utilized to issue

67     flood warnings. EFAS employs a threshold exceedance approach for issuing

68     warnings Bartholmes et al. (2009), where continuous discharge time series

69     are converted into binary series of exceedance or not-exceedance over a dis-

70     charge threshold (the 5-year return period).  In their work, Bartholmes et

71     al. Bartholmes et al. (2009) developed a notification criteria based on two

72     types of persistence: persistence in a particular forecast is determined by the

73     number of NWP members predicting the exceedance, while persistence over

74     several forecast establishes that a flood signal is reliable when consecutive

75     forecast initializations predict the event.  In this study, we will use the term ”probabil

76     ity of exceedance” to refer to the first type of persistence, as the number of

77     NWP members exceeding the discharge threshold can be translated into a

78     probability, assuming equiprobability among them. We will keep the term

79     ”persistence” to denote only the consistency among consecutive forecast initializations.

80	The criteria applied in EFAS to issue flood warnings have changed over the past years  driven by experience of the forecasters and feedback from users. Currently, a 30\% probability threshold and a persistence of 3 consecutive forecasts are used. These cri-

83     teria are independently applied to the two deterministic and the two proba-

84     bilistic NWP models, issuing a notification only if at least one of the models

85     of each type predicts the flood.  Additionally, EFAS notifications are exclu-

86     sively dispatched to locations with a minimum catchment area of 2000 km²

87     and for lead times longer than 48 hours. The first limit is based on the infe-

88     rior hydrological performance in smaller catchments, while the second limit

89     is a consequence of EFAS addressing early awareness and serving as a supplementary system to national or

90     regional services. Previous studies have examined the skill of EFAS notifica-

91     tions Bartholmes et al. (2009) and the response of end users Demeritt et al.

92     (2013). However, the continuous advances in NWP performance Bauer et al.

93     (2015), as well as in the resolution and representation of hydrological pro-

94     cesses in EFAS Joint Research Centre - European Commission (2020, 2023),

95     requires a new evaluation of the notification criteria to harness the potential

96     of the system.

97	The objective of this analysis is to assess the skill of a continental flood

98     forecasting system like EFAS in its current state, and explore new notifi-

99     cation criteria that can maximize its warning skill.  In particular, the first

100     goal is to determine the value of combining several NWP models of different

101     nature in a single system, specifically seeking the appropriate combination

102     method to build a grand ensemble. The second goal is to optimize the two

103     notification criteria —probability threshold and persistence—, analysing the

104     interplay between them and reconsidering whether the persistence criterion is

105     meaningful in a purely probabilistic approach. The third and final goal is to

106     analyse the evolution of the notification skill with catchment area to discern

107     if the newer, higher resolution models are skillful in smaller catchments.

108	To address our research questions, we conducted an analysis of EFAS v.4

109     discharge simulations for the period 2020-2023.   Section 2 enumerates the

110     datasets used in the analysis. Section 3 explains the two experiments that

111     analyse the skill of individual NWP models and different combinations of

112     those models, the criteria optimization process, and the skill metrics we tar-

113     geted.   Section 4 follows a similar structure, presenting first the skill of in-

114     dividual NWP, followed by that of the combination methods, and ends with

115     an exploration of the relationship between notification skill and catchment

116     area. Section 5 discusses the importance of the spatio-temporal framework

117     and target metric in skill analysis like the one here presented, the trade-offs

118     between the notification criteria, the benefits of combining NWP models,

119     and the challenges in selecting a combination method. Finally, Section 6 states the conclusions.

120     2.   Data

121	The data used in this analysis is taken from the operational setup of the

122     EFAS in its version 4 Joint Research Centre - European Commission (2020).

123     EFAS generates a hydrological forecast twice a day (00 and 12 UTC) forced

124     by the meteorological forecast of four different NWP models outlined in Table

125     1. EFAS integrates forecasts from probabilistic models (COS and ENS) and

126     deterministic models (DWD and HRES) to issue flood warnings.

127	The study period spans from the release of EFAS4 in October 2020 until

128     June 2023. For this period we use the EFAS forecasts as the discharge pre-

129     diction and the EFAS reanalysis as a proxy of discharge observations. Using

Table 1: Characteristics of the numerical weather prediction models used in EFAS4.

| Model | Provider | Acronym Max. No. Spatial lead time ensembles resolution |
| --- | --- | --- |
| COSMO-LEPS |  | COS 5.5 days 20 ∼ 7 km |
| ICON-EU/ICON | DWD | DWD 7 days 1 ∼ 6.5-13 km |
| HRES | ECMWF | HRES 10 days 1 ∼ 9 km |
| ENS | ECMWF | ENS 10 days 51 ∼ 18 km |

130     reanalysis as ground truth addresses challenges driven by limited observational hydrological data (e.g.

131     gaps in the observed time series, difference in recorded period etc.), and removes

132     model representation and calibration errors from the analysis. The previous

133     data sets were converted from a continuous variable —discharge— into bi-

134     nary events of exceedance or not exceedance over a threshold. Following the

135     current EFAS setup, the flood threshold is the discharge associated with the

136     5 year return period, which was computed by fitting a Gumbel distribution

137     to the annual maxima from the discharge simulated in the EFAS4 long-term

138     run (1991-2023).

139	In contrast to the original EFAS skill assessment Bartholmes et al. (2009),

140     the current analysis focused solely on real gauging stations rather than all

141     river cells in the model, which would have cause an autocorrelation issue.

142     A selection was made from the approximately 4000 gauging stations in the

143     EFAS database based on two conditions: the contributing area must ex-

144     ceed 500 km², and the hydrological performance, in terms of the modified KGE

145     (Kling-Gupta  efficiency) Kling  et  al.  (2012);  Gupta  et al.  (2009);  Knoben

146     et al. (2019), must not fall below 0.50. The lower bound on catchment was

147     set due to the relatively coarse spatial resolution of NWP and EFAS temporal

148     resolution (6 h), which limits hydrological performance in small catchments.

149     The condition based on hydrological performance ensures the validity of us-

150     ing model simulations as proxies for observations is valid. Ultimately, 1979

151     gauging stations were included in the analysis, accounting for a total of 1683

152     events (exceedances over the 5 year threshold) during the study period. As

153     explained in section 3.4, the optimization of the notification criteria was con-

154     ducted on a subset of 1239 stations with a contributing area larger than 2000

155     km² (874 events), while the entire set was used to analyze the influence of

156     catchment area on warning skill.

|  | predi True | cted False |
| --- | --- | --- |
| True | 1 | 2 |
| observed False | 0 | - |

a)

d)

b)

𝑃𝑟𝑜𝑏𝑎𝑏𝑖𝑙𝑖𝑡𝑦 ≥ 0.5

𝑃𝑒𝑟𝑠𝑖𝑠𝑡𝑒𝑛𝑐𝑒 = 1/1

c)

𝑓.0.8 = 0.56

Figure 1: Overview of the skill assessment framework. a) Hydrographs comparing observed and forecasted discharge (boxplot) with the discharge associated to the 5-year return period (Q5). b) Matrix of total exceedance probability (white dots identify pairs of date and lead time that meet the notification criteria, whereas black rectangles delineate the location of the forecast hydrograph from panel (a) in the probability matrix) and identification of predicted events for specific values of probability threshold and persistence. c) Confusion matrix for the selected station, constructed based on the previous notification criteria. d) Evaluation of the f-score across all combinations of probability threshold and persistence; the black square highlights the skill score corresponding to the notification criteria in panel (b).

157     3.  Methods

158	The skill assessment comprises four steps, as depicted schematically in

159     Figure 1: (a) transformation of both observed and forecasted discharge time

160     series into corresponding time series of the probability of exceeding the 5-

161     year return period; (b) construction of an exceedance probability matrix that

162     consolidates the overlapping forecasts, followed by identification of predicted

163     events through the application of specific notification criteria on that matrix;

164     (c) calculation of the contingency table —hits, misses and false alarms—, (d)

165     evaluation of the skill of the system across all possible combinations of the

166     notification criteria.

167     3.1. Flood detection as exceedance over threshold

168	The notification process starts by converting the continuous time series

169     of the target variable into time series of exceedance or non-exceedance over

170     a predefined threshold.  Within EFAS, flood notifications are based on dis-

171     charge, and the critical threshold is the discharge associated with a 5-year

172     return period (Q5) generated from a 30-year historical simulation.  While

173     alternative alert systems might rely on disparate variables, such as precip-

174     itation Bouttier and Marchal (2023) or river stage Nevo et al. (2022), and

175     employ different thresholds for defining events, the underlying methodology

176     remains similar.

177	The application of the discharge threshold yields distinct outcomes when

178     comparing a single time series—like observations or deterministic forecasts—with

179     an ensemble forecast. In the former case, the time series is converted into

180     a binary sequence, where 0 represents non-exceedance and 1 denotes ex-

181     ceedance. Conversely, ensemble forecasts lead to a probability time series,

182     with values fluctuating between 0 and 1. Since forecasts —both deterministic

183     and probabilistic— are generated more frequently (every 12 hours) than the

184     forecast horizon, they overlap. To address this, the ensemble forecasts are

185     restructured into an exceedance probability matrix that connects each time

186     step to increasing lead times, as illustrated in panel (b) of Figure 1.   Such

187     a configuration simplifies both the analysis and the subsequent computa-

188     tions. Within this matrix, an individual forecast is represented by a diagonal

189     (black rectangles). The uppermost rows represent shorter lead times, where

190     the probability of accurate prediction generally increases. For instance, in the

191     example in Figure 1, the flood event occurring around July 16 was predicted

192     with a 50% probability five days in advance, increasing close to certainty

193     within two days of the event.

194     3.1.1.  Combination of NWPs

195	One of the particularities of EFAS is the simultaneous use of four NWP,

196     both deterministic and probabilistic. The purpose of using multiple NWP is

197     to leverage the strengths of each model. Thus, a primary aim of this study

198     is to develop an effective method for integrating deterministic and proba-

199     bilistic forecasts.  In practical terms, EFAS forecasts produce four distinct

200     probability matrices, one for each NWP, in contrast to the single matrix de-

201     picted in panel (b) of Figure 1. The challenge lies in devising a methodology

202     to combine these four matrices into a total exceedance probability matrix,

203     thereby enhancing the system’s predictive accuracy compared to the use of

204     a single NWP. We explored three different approaches for calculating the to-

205     tal exceedance probability and compared them with EFAS current approach,

206     which does not incorporate a total probability framework.

207	In the current EFAS procedure, a notification is issued if at least one

208     deterministic and one probabilistic NWP predict the event.  This approach

209     will be referred to as 1 deterministic + 1 probabilistic (1D+1P) from here on.

210     As each model is evaluated independently, the current procedure does not

211     calculate a total probability matrix.  The most straightforward method to

212     compute the total probability is the model mean (MM); it is a simple mean

213     over the four exceedance probability matrices, meaning all models are given

214     equal weight. In other words, the single member of a deterministic model is

215     considered significantly more important than any members of a probabilistic

216     model. An alternative approach would be to assign the same weight to each

217     member, regardless of whether it belongs to a deterministic or a probabilistic

218     model. This approach will be referred to as member weighted (MW). In this

219     approach, each NWP receives a weight relative to the number of members it

220     contains, meaning the probabilistic models have greater influence than the

221     deterministic ones. However, neither of these approaches take into account

222     the performance of the NWP. To address this limitation, the Brier weighted

223     approach (BW) assigns a weight to each model based on its probabilistic

224     skill. The Brier score Brier (1950) —a probabilistic error metric that ranges

225     from 0 to infinite, with 0 being the optimal value— is used as a measure of

226     probabilistic skill. The Brier score is calculated using the equation:

Σ1

T

BS =		(P T

t=1

obs,t

- Ppred,t

)2	(1)

227     where BS is the Brier score of a single station, T is the number of time steps,

228     Pobs,t is the observed probability of exceedance, and Ppred,t is the predicted

229     probability of exceedance at a specific time step.  The results of the BW

230     approach are highly sensitive to how Brier scores are converted into weights.

231     The conversion function must transform the values from a metric whose

232     optimal value is 0 to a weight with an optimal value of 1.   Additionally, it

233     should be exponential to amplify the differences among NWP, considering the

234     extremely low BS values that are characteristic in rare events where both the

235     observed and predicted probabilities are 0 most of the time. Inverse distance

236     weighing (eq. 2) fulfils both conditions. Values of the exponent p from 1 to

Table 2: Contingency table in a binary classification. Eobs is the sum of observed events and Epred the sum of predicted events.

Observed

|  | True | False |
| --- | --- | --- |
| Forecasted True | hit | false alarm Epred |
| False | miss | true negative |
|  | Eobs |  |

237

238

239

9 were tested, and 7 was found to be the optimal value.

Σ

BS−p

w =	BS−p	(2)

The weights in the BW method are specific to each NWP and lead time, whereas in the MM and MW methods they remain stable over lead time.

240     3.2. Contingency table

241	The skill assessment of a warning system involves a binary classification

242     task that compares two time series: the predicted and observed events. Typ-

243     ically, the contingency table (Table 2) is used to summarize performance in

244     this type of classification problem.  Given that floods are rare events, the

245     classification task is highly imbalanced, resulting in the number of true neg-

246     atives being orders of magnitude larger than any of the other terms in the

247     contingency table. Consequently, we must omit the true negatives from both

248     the computation of the contingency table and the selection of the target skill

249     metric not to overestimate skill (Section 3.3).

250	The derivation of the binary time series of observed events is straight

251     forward by application of the discharge threshold (Q5) over the reanalysis

252     time series. Conversely, deriving the binary time series of predicted events

253     involves applying two notification criteria to the matrix of total probability

254     of exceedance (or the individual matrices of exceedance probability for each

255     NWP in the 1D+1P approach). The probability threshold sets the minimum

256     value of the exceedance probability at which a high risk of flooding is con-

257     sidered and a notification must be issued. In the current EFAS setup, the

258     threshold is 30% and applies only to the probabilistic forecasts.  Addition-

259     ally, the persistence criterion was introduced to remove false alarms caused

260     by the erratic behaviour of some NWP Bartholmes et al. (2009).  This cri-

261     terion aims to replicate the behaviour of a forecaster, who would wait to

262

263

264

265

266

267

268

269

270

271

272

273

274

275

276

277

278

279

280

send the warning until consecutive forecasts predict the event. In the cur- rent procedure, notifications are sent if 3 consecutive forecasts exceed the probability threshold (hereafter referred to as 3/3 persistence). Panel (b) in Figure 1 illustrates the application of these two notification criteria on the matrix of exceedance probability. For every time step (columns), the white dots identify the lead times that meet the criteria. Eventually, a flood event is predicted at a particular time step if any of the lead times meet the criteria. The terms of the contingency table result from comparing the predicted and observed events. We consider a hit any observed event in which at least one time step was correctly predicted. The number of misses and false alarms are calculated, respectively, as the difference between the observed (Eobs) or

predicted (Epred) events and the hits (eq. 3).

misses = Eobs − hits	(3)

false alarms = Epred − hits	(4)

The process is repeated for all possible combinations of the two criteria and across all stations. We tested probability threshold values ranging from 5% to 95% and six persistence values: 1/1 (no persistence), 2/4, 2/3, 2/2, 3/4 and 3/3 (the current persistence criterion). The outputs of this process are matrices of hits, misses and false alarms for each station an combination of the probability threshold and persistence that will be used in the subsequent section to compute skill metrics.

281     3.3.  Skill metrics

282	The fact that the classification task we face is highly imbalanced is fun-

283     damental in the selection of the skill metric from the myriad of options. For

284     instance, the odds ratio and the Hanssen-Kuipers score (HK) Hanssen and

285     Kuipers (1965) were discarded because both include the true negatives, which

286     is a term to be excluded in an imbalanced classification.  Instead, we have

287     selected three skill metrics specific for imbalanced classification, i.e., that

288     exclude the true negatives.

289	Recall is the proportion of observed events that are correctly predicted

290     (eq. 5); in some contexts it is also known as hit rate, probability of detection

291     or sensitivity. Bartholmes Bartholmes et al. (2009) proved that recall is equal

292     to HK for highly imbalanced classifications, such as floods. Precision, a.k.a.

293     positive predicted value or frequency of hits, is the proportion of the predicted

294     events that are correct (eq. 6). There is a well known trade-off between recall

295     and precision. In the specific case of flood notifications, very strict criteria

296     would cause very few notifications with high certainty, which means that they

297     will be mostly correct (high precision), but a lot of events would be missed

298     (poor recall). On the other hand, very relaxed criteria would cause a large

299     amount of uncertain notifications, which means a small number of missed

300     events (high recall) but a large number of false alarms (poor precision). As

301     a compromise, the fβ score is a metric that balances recall and precision (eq.

302     7). In its most common version (β = 1) it is the harmonic mean of recall

303     and precision, granting equal importance to both terms.   However, higher

304     importance can be given to precision (β < 1), therefore limiting the amount

305     of false alarms, or to recall (β > 1), reducing the amount of misses. In our

306     study, after suggestions from forecasters analyzing EFAS on a daily basis, we

307     selected f0.8 as the target skill metric.  This metric balances precision and

308     recall giving a slightly higher value to the former. The idea behind is to limit

309     the amount of false alarms, which would jeopardize the trust of the recipients

310     of the warnings.  Both recall, precision and the fβ score range from 0 to 1,

311     being 1 their optimal value.

312	As a final validation metric, we used bias, which represents the proportion

313     between the number of predicted and observed events and is calculated as

314     the quotient between recall and precision (eq.   8).   Its optimal value is 1,

315     which means that the number of notifications issued is equal to the number

316     of observed events, and it ranges from 0 to infinity. All four metrics presented

317     here can be plotted in the Roebber diagram Roebber (2009). We modified

318     the original Roebber diagrams and replaced the critical success index by our

319     target metric (f0.8).

320

recall =

precision =

hits

=

Eobs

hits

=

hits

hits + misses

hits

(5)

(6)

321

Epred

hits + false alarms

322

f  =  1 + β2	precision · recall

β

β2 · precision + recall

(7)

bias = Epred

hits + false alarms	recall

=	=

(8)

323

Eobs

3.4. Optimal notification criteria

hits + misses

precision

324	The current EFAS notification criteria involve 5 variables:  minimum

325     catchment area, minimum lead time, combination of NWP, probability thresh-

326     old and persistence. We analysed and optimized all these variables, initially

327     isolating each one to understand its individual effects.

328	We have conducted two main experiments to explore the benefits of com-

329     bining NWP. The first experiment analyzes each NWP individually with two

330     objectives: identifying the strengths and weaknesses of each NWP, and un-

331     derstanding how the notification criteria affect differently probabilistic and

332     deterministic NWPs.  The second experiment compares the four combina-

333     tions of NWP explained in section 3.1.1.  The aim is to identify the most

334     appropriate combination method and compare it against the most skillful

335     NWP to assess whether the combination of NWP adds value to the system.

336     The setup of these two experiments is similar, apart from the difference in

337     the input data, and is explained in the following paragraphs.

338	For the majority of the analyses, we used only stations that met the

339     current minimum catchment area of 2000 km². With this subset of stations,

340     we explored the evolution of skill with lead time, probability and persistence.

341     The 20 lead time values —2 forecasts per day and an horizon of 10 days—

342     were grouped in two ways.  First, we grouped them daily to analyze the

343     evolution of skill with lead time.	Second, to explore the impacts of the

344     probability threshold and persistence, we defined 4 lead time ranges:   a)

345     shorter than 2 days, a range at which EFAS formal notifications cannot be

346     issued, b) 2-5.5 days, when all NWP are available, c) 5.5-7 days, when 3

347     NWP are available as the maximum lead time of COS is exceeded, d) 7-10

348     days when only 2 NWP are available as the maximum lead time of DWD is

349     exceeded.

350	Following the exploration, we proceeded to optimize the probability thresh-

351     old and the persistence criteria. Specifically, we sought the combination of

352     probability and persistence that produces the maximum f0.8 at each of the

353     4 lead time ranges previously specified. The optimization was repeated for

354     every NWP individually and for every combination of the NWP. To ensure

355     a robust selection of criteria, we applied a 10-fold cross validation: 20% of

356     the stations were left aside as a test set, while the remaining 80% were sub-

357     divided in 10 folds. We computed the skill for every combination of 9 folds

358     and then averaged over folds. The optimal notification criteria were derived

359     from this average-over-fold skill matrix, as shown in panel (d) of Figure 1).

360     Subsequently, the selected criteria were evaluated on the test set to prevent

361     overfitting.

362	Lastly, we explored the impact of catchment area on warning skill using

363     the entire set of stations. We calculated the notification skill for increasing

364     values of the minimum catchment area, ranging from 500 km² to 300,000

365     km², using the criteria previously optimized for the minimum catchment

366     area of 2000 km². Subsequently, we tested whether the skill of the system,

367     particularly that of small catchments, significantly improves with a more

368     complex notification criteria in which the probability threshold varies with

369     catchment area. We fixed the persistence criterion to that obtained in the

370     initial optimization and only tuned the probability threshold for increasing

371     values of the catchment area threshold. We compared the skill of the fixed

372     probability threshold with the skill of the area-specific probability threshold

373     to evaluate whether the new, more complex criteria pays off in terms of

374     performance.

375     4.  Results

376     4.1. Analysis of ”observed” events

377	Figure 2 depicts the spatial distribution of the 1239 stations used in the

378     optimization of the notification criteria, i.e., those with a minimum contribut-

379     ing area of 2000 km² and a KGE not lower than 0.50. The colours represent

380     the amount of ”observed” flood events, while the histogram at the bottom

381     shows the frequency of events across the stations.

382	In total, the optimization of the notification criteria will be based on 1239

383     stations and 874 ”observed events”. A higher concentration of points with

384     events is observed in Central Europe, the British Isles, and Mediterranean

385     catchments.	Significant flood events in the Rhine, Meuse, and Ebro are

386     visually evident on the map.	Throughout the study period,  61% of the

387     stations did not exceed the 5 year return period, which means that hits and

388     misses can not exist, and the only term in the contingency table will be false

389     alarms. Consequently, the f-score for these stations can be either 0 or null.

390     Additionally, 29% of stations registered only one ”observed” event.   These

391     nuances bear implications for the overall skill of the system, as we will explore

392     in subsequent discussions.

393	The imposition of a minimum contributing area of 2000 km² places a

394     constraint on the number of stations available for optimization, consequently

395     affecting the count of observed events. In the final phase of our analysis, we

396     assessed the implications of this criterion by expanding our consideration to

397     stations with a minimum area of 500 km². Figure 3 represents the relation-

398     ship between an increasing catchment area threshold and the corresponding

no. points

1000

100

10

1

0

1	2	3	4	5	6	7	8	9	10   11   12   13

no. observed events

Figure 2: Geographical distribution of the stations used for optimizing the notification criteria, and number of ”observed” flood events during the study period.

points

events

2000

1500

count (-)

1000

current limit

500

0

103

104	105

area (km²)

Figure 3: Number of stations (orange) and observed events (blue) by catchment area threshold. The vertical, dotted line represents a catchment area of 2000 km², the current limit in the EFAS notifications.

399     number of stations and ”observed” events. Both counts decrease exponen-

400     tially with catchment area.  By excluding the points with contributing area

401     smaller  than 2000 km², we  discarded 37% of the stations and 48% of the

402     events.   The objective of this decision was to use the current notification

403     criteria as benchmark.

404     4.2.  Individual NWP

405	Prior to studying how to combine the NWP models, we analysed the

406     skill of each of the models individually. This analysis serves a dual purpose.

407     First, to understand the relative skill of each model, with the intention of

408     incorporating this skill into the computation of the total probability. Second,

409     to compare the skill of individual NWP models against the benchmark—

410     the current notification criteria—, and to set a baseline for the combination

411     methods.

412     4.2.1.  Probabilistic skill assessment

413	In the initial assessment of NWP skill, we calculated the Brier score for

414     each NWP at daily lead times.  The Brier score, as an error metric, yields

415     values that can be challenging to interpret, given the dependence of error

416     magnitude on the specific variable—particularly in the context of rare events

417     like floods, where the magnitude is inherently low. Instead, we employ the

1.0

Brier skill score (-)

0.8

ENS COS HRES DWD

start notif.

end COS

end DWD

0.6

0.4

0.2

0.0

0	2	4	6	8

lead time (d)

Figure 4: Brier skill score of the four NWP used in EFAS at daily lead times. The reference (BSS = 0) is a model that never predicts an event. Blue lines represent probabilistic models, and orange lines deterministic ones.

418     Brier skill score (BSS) to gauge the relative skill of a model in comparison

419     to a reference. Models outperforming the reference receive a positive BSS,

420     while those underperforming register a negative BSS. For the reference, we

421     opted for a dummy model that never predicts an event (Ppred = 0) Legg and

422     Mylne (2004).

423	The plot above demonstrates that probabilistic models exhibit greater

424     skill than deterministic ones.  Only within very short lead times (0-1 days)

425     do deterministic models approach the skill observed in probabilistic models.

426     This is particularly significant for EFAS notifications,  as the formal noti-

427     fications are reserved for lead times greater than 2 days (indicated by the

428     leftmost dotted line). As lead time increases, there is a degradation in skill.

429     This decline impacts ENS to a lesser extent than the other models. Both de-

430     terministic models display poor skill at their forecast horizon. For instance,

431     at 10 days lead time, the skill of HRES is worse than a model that never

432     predicts a flood.

433	Overall, the most skillful model for flood warning forecasting is ENS,

434     while the least skillful is DWD. In section 4.3, we will convert the Brier score

435     into weighing factors for the computation of total probability (see Figure 8).

436     4.2.2.  Notification skill

437	In this section we analysed how the NWP would have performed if notifi-

438     cations would be sent based uniquely in one of them. First, we illustrate the

439     general behaviour of notification skill according to the three dimensions here

1.0

0.8

f0.8 (-)

0.6

0.4

0.2

start notif.

end COS

end DWD

COS

DWD

ENS

HRES

current

P    0.10

P    0.20

P    0.30

P    0.40

P    0.50

P    0.60

P    0.70

P    0.80

P    0.90

0.0

0	2	4	6	8

0	2	4	6	8

0	2	4	6	8

0	2	4	6	8

lead time (d)

lead time (d)

lead time (d)

lead time (d)

Figure 5: Evolution of the notification skill with daily lead times and probability threshold for each NWP model in an scenario with no persistence. Each plot represents a different NWP. As a benchmark, the black, solid line represents the skill of the current operational criteria (1 deterministic + 1 probabilistic, 30% probability and a persistence of 3 forecasts).

440     involved —lead time, probability threshold and persistence.   Second, we

441     optimize the probability and persistence criteria for a fixed lead time range.

442	Figure 5 exhibits the evolution of the notification skill with lead time and

443     probability threshold for a scenario with no persistence. The black, solid line

444     is the notification skill of the current notification criteria, as a benchmark.

445     Each colour line represents a different probability threshold; orange lines

446     represent thresholds below 50% and blue lines over that value. For the two

447     deterministic models (DWD, HRES) all lines overlap, since the probability

448     threshold does not affect a deterministic forecast.

449	In line with Figure 4, the probabilistic models show overall higher skill than the

450     deterministic models.   Under certain conditions, they even outperform the

451     current notification criteria, indicating potential for enhancing the skill of

452     the current EFAS notification criteria.  Deterministic models demonstrate

453     skill on par with the current criteria solely for the initial lead time.  As ex-

454     pected, notification skill degrades with lead time.  This trend is consistent

455     across all cases —deterministic, probabilistic and benchmark—, although the

456     rate of degradation depends on the probability threshold. Lower probability

457     thresholds sharply degrade at short lead times, while high probability thresh-

458     olds experience a more pronounced loss of skill at longer lead times (such as

459     ENS over 7 days).

460	There is a range of probability thresholds, approximately from 40% to

461     90%, exhibiting similarly good skill.   Within this range, higher thresholds

462     perform slightly better at short lead times, and lower thresholds perform

463     slightly better at mid-long lead times. As we are seeking to identify a single

464     optimal probability threshold for the notification criteria, the selection of the

1.0

0.8

f0.8 (-)

0.6

0.4

COS

DWD

ENS

HRES

current

1/1

2/4

2/3

2/2

3/4

3/3

0.2

0.0

0.2

0.4      0.6      0.8

probability

0.2

0.4      0.6      0.8

probability

0.2

0.4      0.6      0.8

probability

0.2

0.4      0.6      0.8

probability

Figure 6: Evolution of the notification skill with probability threshold and persistence for each NWP model at a fixed lead time range (from 2 to 5.5 days). Each plot represents a different NWP model. As a benchmark, the black cross represents the current criteria (1 deterministic + 1 probabilistic, 30% probability and a persistence of 3 forecasts).

465     optimal criteria will prioritize a lead time range from 2 days (the minimum

466     formal notification lead time) to 5.5 days (the maximum lead time in COS).

467     This approach allows us to mitigate the substantial uncertainty associated

468     with longer lead times, and focus on the lead times at which the majority of

469     formal notifications are issued and actions are taken from first responders.

HERE WE NEED A NEW SUB-SUBSECTION TO MAKE IT EASIER TO READ

470	Up to this point, we have not yet considered persistence in our analysis.

471     In order to assess the impact of persistence on the notification skill,

472     Figure 6 illustrates the evolution of skill with the

473     probability threshold and persistence for a fixed lead time range (2-5.5 days).

474     The benchmark (current notification criteria) is now represented by a single

475     point corresponding to a 30% probability and a persistence of 3/3. As the

476     probability threshold does not affect deterministic forecasts, the lines for the

477     two deterministic models (DWD and HRES) are horizontal.

478	Three out of four NWP models are able to replicate or improve the skill

479     of the current notification criteria at certain combinations of persistence and

480     probability.   Only DWD is less skillful than the benchmark.   The skill of

481     the probabilistic models shows a trade-off between probability threshold and

482     persistence. Both criteria play the same role of removing false positives (or

483     uncertain events) by taking stricter values. Consequently, stricter persistence

484     necessitates lower probability thresholds to optimize notification skill, and vice versa.

485     Nevertheless, the highest skill are consistently obtained in the scenario of no

486     persistence (1/1). In this scenario, there is a wide range of probability thresholds

487     (from 40 to 90%) that perform similarly well.

488	Persistence does have a significant impact in the deterministic models.

489     Their skill improves dramatically as soon as some persistence is added in the

Table 3: Summary of the optimization of the notification criteria for individual NWP and four lead time ranges. The initial row serves as a benchmark, indicating the skill of the current notification criteria. In each lead time range, bold fonts highlight the maximum obtained skill for a specific metric.

| lead time | model | probability | persistence | recall | precision | f0.8 | rank |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <2 d | 1D+1P | 0.300 | 3/3 | 0.522 | 0.735 | 0.634 | 3 |
|  | COS | 0.875 | 1/1 | 0.673 | 0.718 | 0.700 | 1 |
|  | DWD | - | 2/2 | 0.585 | 0.583 | 0.583 | 5 |
|  | ENS | 0.800 | 1/1 | 0.681 | 0.711 | 0.699 | 2 |
|  | HRES | - | 2/2 | 0.580 | 0.671 | 0.632 | 4 |
| 2-5.5 d | 1D+1P | 0.300 | 3/3 | 0.413 | 0.618 | 0.518 | 3 |
|  | COS | 0.725 | 1/1 | 0.387 | 0.687 | 0.518 | 2 |
|  | DWD | - | 3/3 | 0.420 | 0.522 | 0.477 | 5 |
|  | ENS | 0.650 | 1/1 | 0.416 | 0.725 | 0.562 | 1 |
|  | HRES | - | 3/3 | 0.416 | 0.612 | 0.517 | 4 |
| 5.5-7 d | 1D+1P | 0.300 | 3/3 | 0.215 | 0.612 | 0.356 | 2 |
|  | DWD | - | 2/2 | 0.291 | 0.358 | 0.328 | 4 |
|  | ENS | 0.450 | 1/1 | 0.264 | 0.605 | 0.402 | 1 |
|  | HRES | - | 2/2 | 0.273 | 0.429 | 0.351 | 3 |
| 7-10 d | 1D+1P | 0.300 | 3/3 | 0.120 | 0.625 | 0.237 | 3 |
|  | ENS | 0.175 | 2/2 | 0.287 | 0.396 | 0.345 | 1 |
|  | HRES | - | 2/2 | 0.244 | 0.336 | 0.293 | 2 |

490     criteria. For the most stringent persistence (3/3), the skill of the determin-

491     istic HRES is as good as that of the probabilistic ENS, and even better than

492     COS. However, as mentioned earlier, this is a sub-optimal value of persistence

493     for the probabilistic models.

494	With the knowledge gained in the previous exploration, we were in a bet-

495     ter position to understand the outcome of optimizing the notification criteria

496     individually for each NWP, summarized in Table 1. The results are organised

497     by lead time ranges; for each range, the table presents the optimized criteria

498     —probability and persistence— and the skill metrics —recall, precision and

499     f0.8— for the available NWP models. As a benchmark, the first row in each

500     lead time range shows the skill of the current operational criteria.

501	The current EFAS setup (1D+1P) notifies 52% of the actual events (re-

502     call) and 74% of the notifications are correct (precision) at most.    These

503     figures reflect a negative bias, i.e., notifications are issued with a high level

504     of certainty, leading to a limited number of false alarms, but many events

505     being missed.   This behaviour aligns with the selection of a biased target

506     metric, such as the f0.8 score, for the optimization of the notification criteria.

507     The current criteria is not optimal as 1D+1P is consistently outperformed in

508     terms of f0.8 at any lead time by single NWP, suggesting that the combination

509     of all NWPs hinders the skill ot that single NWP.

510	At the shortest lead time range, COS emerges as the top-performing

511     NWP, closely followed by ENS. Both ENS and COS optimized a very high

512     probability threshold and removed persistence. On the other hand, the skill

513     of the two deterministic NWP is distinctively lower —particularly DWD—,

514     and both require a persistence of 2/2.   The current operational criteria is

515     outperformed by any of the probabilistic models, and the deterministic HRES

516     almost reaches that benchmark.

517	The scenario remains similar when examining lead times from 2 to 5.5

518     days. The two probabilistic NWPs continue to demonstrate higher skill than

519     the deterministic ones, and outperform the current operational criteria. In

520     this case, ENS is the top-performing NWP. Neither of them require persistence

521     and the probability thresholds are lower than in the previous lead time range,

522     although still higher than the 30% value used currently. HRES, despite being

523     deterministic, almost matches the skill of the benchmark and COS. As well

524     as DWD, to maximize its skill requires a persistence of 3/3.  Overall, there

525     is a loss in skill of approximately 20% compared with the first 2 days of

526     forecast.

527	At lead times from 5.5 to 7 days, ENS

528     continues to outperform the other models.   The optimal criteria follow the

529     trend: no persistence and a decreasing probability threshold, though still

530     higher than the benchmark. Overall skill has degraded by 30% compared

531     to the previous lead time range (over 40% compared to the first 2 days of

532     forecast).

533	The skill at the longest lead time range is overall poor. Only two models

534     have such a long forecast horizon, with ENS demonstrating significantly bet-

535     ter skill.  This long lead time range is the only one for which ENS requires

536     persistence (2/2), a factor that notably decreases the probability threshold,

537     reflecting the trade-off between these two criteria.

538	The Roebber diagram in Figure 7 offers an alternate perspective on the

539     same results. Robber diagrams condense four skill metrics into a single plot

540     Roebber (2009), with an ideal model positioned at the top right corner.

541     Precision and recall are indicated on the X and Y axes, respectively. The

542     slope of the dashed lines represents bias, calculated as the ratio of recall

1.0

7.6

3.5

bias

2.1

1.4

0.8

recall

0.6

0.4

0.2

0.0

0.6

0.8

0.6

0.4

0.2

0.4

0.2

0.1

0.0	0.2	0.4	0.6	0.8	1.0

precision

current

NWP

bias

ENS COS HRES DWD

COMB

BW MW MM 1D+1P

Figure 7: Roebber diagram of the optimized criteria at the lead time range from 2 to 5.5 days. Four skill metrics are represented:  precision and recall in the X and Y axis, bias by the dashed lines, and the f0.8 score by the solid lines. Triangles illustrate individual NWP and circles the combination of NWP. For reference, the skill of the current notification criteria is shown as a black cross.

543     to precision.	Points below the 1:1 line indicate negative bias (a smaller

544     number of notifications than observed events), while points above the line

545     indicate positive bias (an excessive number of notifications). The f0.8 score

546     is represented by solid, black lines. Figure 7 shows simultaneously the skill

547     of the current notification criteria, the optimization of individual NWP, and

548     the optimization of combination methods (refer to Section 4.3) for the lead

549     time range 2-5.5 days.

550	All NWP models and the current criteria exhibit negative bias, indicating

551     that EFAS underpredicts the occurrence of flood events. In the case of the

552     optimizations carried out in this analysis, the selection of the f0.8 score as the

553     target metric in the optimization forces this bias, as it prioritizes precision

554     over recall. However, the current setup already shows a negative bias. It is

555     remarkable that the improved f0.8 of the probabilistic models is a result of

556     higher precision, while recall values remain similar across all NWPs and the

557     current criteria.

558     4.3. Combination of models

559	As explained in the methodology, three of the four combination methods

560     compute total probability, which requires assigning weights to each NWP

561     and lead time. Figure 8 illustrates the distribution of weights among NWP

562     models for these three combination methods, and how the weights vary with

1.0

cumulative weight (-)

0.8

0.6

model mean

member weighted

Brier weighted

DWD HRES COS ENS

0.4

start notif.

end COS

end DWD

0.2

0.0

0	2	4	6	8

0	2	4	6	8

0	2	4	6	8

lead time (d)

lead time (d)

lead time (d)

Figure 8: Weights applied to compute the total probability. Every plot represents a different methods, with colours representing NWPs (blue for probabilistic and orange for deterministic models).

563     lead time.

564	The model mean (MM) approach attributes equal weights to each model.

565     The changes in the weights with lead time occur when the maximum lead

566     time of a model is exceeded (5.5 days for COS and 7 days for DWD). This is

567     the approach in which the deterministic models are given equal importance

568     as the probabilistic models.

569	Instead, in the member weighted (MW) combination, every model run

570     is assigned an equal weight. Therefore, ENS, which has a significant larger

571     number of members, prevails over any other NWP. Even for the first 5 days,

572     when all the models are available, ENS receives 70% of the weight.  Con-

573     versely, the sum of the two deterministic models (HRES and DWD) has a

574     maximum weight of 4% from days 5.5 to 7. In general, this approach heavily

575     relies on the skill of the largest ensemble, which is not necessarily the most

576     skillful, as is in the case of EFAS.

577	In the Brier weighted (BW) combination, we used the Brier scores from

578     section 4.2.1 to derive weighing factors that prioritize the more skillful mod-

579     els.  The resulting weights vary with lead time, showing a transition from

580     short to long lead times. At very short lead times, the deterministic models

581     demonstrate skill, accounting for approximately a third of the total weight.

582     However, as the lead time increases, the probabilistic models, particularly

583     ENS, assume the majority of the weight. At lead times from 2 to 5.5 days,

584     when most of the notifications are sent, the superior skill of probabilistic

585     models is represented by a combined weight of approximately 80%.

1 deterministic + 1 probabilistic

start notif.

end COS

end DWD

1.0

0.8

f0.8 (-)

0.6

0.4

0.2

model mean

member weighted

brier weighted

ENS

P    0.10

P    0.20

P    0.30

P    0.40

P    0.50

P    0.60

P    0.70

P    0.80

P    0.90

0.0

0	2	4	6	8

0	2	4	6	8

0	2	4	6	8

0	2	4	6	8

lead time (d)

lead time (d)

lead time (d)

lead time (d)

Figure 9: Evolution of the notification skill with daily lead times and probability threshold for each of the combination methods in a scenario with no persistence. Each plot rep- resents a different combination of NWP. As a benchmark, the black, solid line represents the skill of ENS with optimized criteria.

586     4.3.1.  Notification skill

587	Figure 9 and Figure 10 replicate the analysis of the notification skill previ-

588     ously done for the individual NWP models, but this time for the combination

589     methods. Figure 9 illustrates the evolution of the notification skill with lead

590     time and probability threshold for a fixed persistence value (no persistence).

591     On the other hand, Figure 10 presents the evolution of skill with probability

592     and persistence for a fixed lead time range (2-5.5 days). In both cases, the

593     benchmark is ENS, the NWP that proved highest skill.  Together these fig-

594     ures an overview of the behaviour of skill across four dimensions: combination

595     method, lead time, probability threshold and persistence.

596	The results indicate that ENS is responsible for the majority of the skill

597     in the grand ensembles, since only marginal gains are possible with specific

598     combinations at certain lead times.  Only two combination methods (MW

599 and BW) are able to replicate the skill of the benchmark across all lead 600      times, with minimal improvements at days 2, 4 and 6.   Although MM at 601 lead time 0 achieves the highest marginal gain, this analysis proves that this 602 combination method is not suitable, as the skill of this grand ensemble is 603     poorer than the benchmark from day 2 onwards.   The current procedure 604 (1D+1P) is also sub-optimal, even though the degradation with lead time is 605 not as severe as with MM.

606	Regardless of the NWP or combination of NWPs, the notification skill

607 degrades notably with lead time. Even if at lead time 0 the skill is high (up 608 to 0.74), the minimum lead time criterion limits the EFAS notification skill 609 to values slightly lower than 0.6. The degradation continues and, from day 7 610    onward, the skill drops below 0.4, which may be an indicator for creating a

1 deterministic + 1 probabilistic

1.0

0.8

f0.8 (-)

0.6

0.4

model mean

member weighted

brier weighted

ENS

1/1

2/4

2/3

2/2

3/4

3/3

0.2

0.0

0.2      0.4      0.6

probability

0.8

0.2      0.4      0.6

probability

0.8

0.2      0.4      0.6

probability

0.8

0.2      0.4      0.6

probability

0.8

Figure 10: Evolution of the notification skill with probability threshold and persistence for each combination of NWP at a fixed lead time range (from 2 to 5.5 days). Each plot represents a different NWP model. Each plot represents a different combination of NWP. As a benchmark, the black cross represents the skill of ENS with optimized criteria (65% probability and no persistence).

611  new limitation on the maximum lead time at which notifications are issued. 612	Figure 10 reinforces the idea that the persistence criterion limits the 613 maximum attainable skill in ensemble systems.  Across all four combina- 614 tions, the highest skill is reached when no persistence is applied (1/1). In 615    this scenario, all combinations exhibit similar maximum skill. Only member 616 weighted (MW) and Brier weighted (BW) marginally surpass the baseline 617    (ENS). These two combinations show optimal skill within a range of proba- 618 bility thresholds spanning 40% to 70%. In both instances, the maximum skill 619 is achieved at a probability threshold lower than that optimized for ENS.

620	The absence of sensitivity to probability in the current criteria (indicated

621     by the dark blue line in the left-hand side pane) is noteworthy.   Even at 622 the lowest probability threshold, the f0.8 score remains at 0.5, unlike other 623   approaches where there is a significant decline in performance when the prob- 624 ability threshold takes very low values.

625	Table 4 summarizes the results of the optimization of the notification 626 criteria for the combination methods in an identical way as Table 3 did for 627 the NWPs. The results are organised by lead time ranges; for each range, the 628   table presents the optimized criteria (probability and persistence) and the 629 skill metrics (recall, precision and f0.8) for the four methods. As a baseline, 630 the first row in each lead time range shows the skill of ENS. The objective 631 of the optimization is again two-fold: to compare the four combinations of 632 NWP models, and to assess the added value of building a grand ensemble 633 instead of simply using the baseline.

634	In the shortest lead time range, two combinations clearly outperform

635    the baseline (MM, BW). Between the two, MM exhibits the highest skill

Table 4: Summary of the optimization of the notification criteria for the combination methods and four lead time ranges. The initial row serves as a baseline, indicating the skill of ENS, the top-performing NWP. In each lead time range, bold fonts highlight the peak value for a specific metric.

| lead time | method | probability | persistence | recall | precision | f0.8 | rank |
| --- | --- | --- | --- | --- | --- | --- | --- |
| <2 d | ENS | 0.800 | 1/1 | 0.681 | 0.711 | 0.699 | 4 |
|  | 1D+1P | 0.925 | 1/1 | 0.662 | 0.717 | 0.694 | 5 |
|  | MM | 0.775 | 1/1 | 0.628 | 0.838 | 0.741 | 1 |
|  | MW | 0.750 | 1/1 | 0.673 | 0.718 | 0.700 | 3 |
|  | BW | 0.950 | 1/1 | 0.620 | 0.829 | 0.733 | 2 |
| 2-5.5 d | ENS | 0.650 | 1/1 | 0.416 | 0.725 | 0.562 | 3 |
|  | 1D+1P | 0.600 | 1/1 | 0.455 | 0.643 | 0.554 | 5 |
|  | MM | 0.700 | 1/1 | 0.424 | 0.700 | 0.559 | 4 |
|  | MW | 0.500 | 1/1 | 0.455 | 0.692 | 0.574 | 2 |
|  | BW | 0.525 | 1/1 | 0.451 | 0.700 | 0.576 | 1 |
| 5.5-7 d | ENS | 0.450 | 1/1 | 0.264 | 0.605 | 0.402 | 1 |
|  | 1D+1P | 0.425 | 1/1 | 0.262 | 0.599 | 0.399 | 4 |
|  | MM | 0.500 | 1/1 | 0.276 | 0.412 | 0.345 | 5 |
|  | MW | 0.425 | 1/1 | 0.271 | 0.579 | 0.401 | 2 |
|  | BW | 0.450 | 1/1 | 0.268 | 0.586 | 0.400 | 3 |
| 7-10 d | ENS | 0.175 | 2/2 | 0.287 | 0.396 | 0.345 | 1 |
|  | 1D+1P | 0.250 | 1/1 | 0.256 | 0.414 | 0.334 | 4 |
|  | MM | 0.225 | 2/2 | 0.252 | 0.334 | 0.296 | 5 |
|  | MW | 0.175 | 2/2 | 0.278 | 0.402 | 0.343 | 3 |
|  | BW | 0.300 | 1/1 | 0.255 | 0.445 | 0.345 | 1 |

636 (f0.8 = 0.741).  This proficiency can be attributed to the importance that 637 MM assigns to deterministic models, which demonstrated notable skill at 638 very short lead times (see Figure 8). As of the optimized criteria, none of 639 the four methods required persistence, and probability thresholds fell within 640 the top quartile.

641	From day 2 to 5.5, BW and MW stand out as the top-performing com- 642     binations, with a comparable skill that surpass the baseline by 1%.   Both 643   approaches operate optimally without the need for persistence, with the prob- 644 ability threshold hovering around 50% (refer to Figure 10 for a sense of the 645   uncertainty in defining the probability threshold). Compared with the pre- 646 ceding lead time range, there is an evident decline in skill by approximately 647 20%.

648	In the third lead time range, from 5.5 to 7 days, none of the combinations

649 outperform the baseline, despite three of them (1D+1P, MW and BW) at- 650 taining very similar skill. None of these three methods necessitated the use 651 of persistence, and the associated probability thresholds were slightly lower 652 than those in the preceding lead time range (around 40%). When compared 653 with the prior lead time range, there is a decrease in skill of around 30% 654 (45% when compared to the the first range).

655	In the longest lead time range, BW emerges as the best-performing com- 656  bination,  with  a  skill  on  par  with  the  baseline.  MW,  while  slightly  less 657    proficient, shows similar skill; however, their optimized criteria differ. MW 658     requires a persistence of 2/2 and a relatively low probability threshold (0.175) 659 — precisely the same criteria optimized for the baseline. This results are not 660  surprising, as at this lead time range MW and ENS are practically the same, 661 since the weight of HRES in the grand ensemble is negligible. In contrast, 662   BW does not require persistence, and its probability threshold (30%) aligns 663 with the decreasing trend observed in previous lead time ranges.

A NEW SUB-SUBSECTION IS NEEDED HERE TO MAKE IT EARIER TO READERS TO FOLLOW

664	The maps in Figure 11 illustrate the skill metrics associated with the

665    optimal notification criteria, i.e., the BW combination with no persistence 666 and a probability threshold of 52.5%.   The metrics represent only the lead 667 time range from 2 to 5.5 days, which corresponds to the period when the 668 majority of the notifications are issued.

669	A pronounced contrast is evident in the maps, attributable to the limited 670       number of events in most stations, resulting in skill metrics primarily yielding 671 values of either 1 or 0. In terms of f-score, there is a clustering of under- 672 performing points in central Europe (notably the Alps) and central France. 673 The breakdown into recall and precision reveals that the predominant cause

f0.8

recall

precision

0	0.2	0.4	0.6	0.8	1.0

skill (-)

-0.5 0

skill (-)

-1

0.5

1

Figure 11: Skill maps for the optimized notification criteria (BW, 52.5% probability and no persistence) at the lead time range from 2 to 5.5 days.  The main plot shows the skill in terms of f0.8 (the target metric); in this plot, gray dots are stations for which the f- score cannot be computed due to all instances of hits, misses and false alarms being null. The smaller plots show precision and recall, the components of the f-score; for the sake of visibility, it only shows the stations for which the specific metric can be computed. In all the plots the direction of the arrows shows the difference in that metric between the optimized criteria and the current criteria; positive values imply gains and negative values losses in skill.

1.0

0.8

skill (-)

0.6

f0.8

recall

precision

current 1D+1P MM MW BW

0.4

min. area

0.2

0.0

103

104	105

area    (km²)

103

104	105

area    (km²)

103

104	105

area    (km²)

Figure 12: Evolution of skill with catchment area limit. Each plot represents a skill metric, with f0.8 as the optimization target, and recall and precision as its components. In each of the plots, the black line represents the skill of the current notification criteria, while the colour lines depict the skill of the optimized criteria for the 4 combination methods. The vertical, dotted line indicates the catchment area limit currently applied in EFAS.

674 of  the low f-score lies  in recall,  indicating  missed events.  Conversely,  the 675 precision of notifications remains consistently high, with exceptions observed 676 in the Seine and Arno rivers. It is noteworthy the skill in all metrics for 677   the Ebro, Rhine and Meuse—three rivers that experienced significant flood 678 events during the study period.

679	By applying this set of criteria, EFAS would be able to issue notifications 680 with at least 2 days lead time for 45% of the events, and 70% of the notifica- 681   tions would be correct. For reference, these figures represent an improvement 682 in both metrics over the existing criteria, which achieved 41% recall and 62% 683   precision (see Figure 7).  The Roebber diagram indicates that the grand en- 684 sembles —namely MW and BW— still suffer from a negative bias, but it was 685 slightly reduced by improving recall compared with ENS.

686    4.4. Identification of optimal catchment area to issue notifications

687	The preceding sections presented results for a set of stations with a catch- 688      ment area of at least 2000 km², the current limit. To investigate the potential 689 relaxation of this criterion, Figure 12 illustrates the evolution of skill depend- 690 ing on the catchment area limit. The criteria used to create this figure are 691    those optimized for the fixed area limit of 2000 km² and a lead time range 692   between 2 and 5.5 days (refer to Table 4). For benchmarking purposes, the 693 plots compare the skill of the four combination methods (coloured lines) with 694 the skill of the current notification criteria (black line).

695	The skill of the system improves with catchment area, as expected. For 696 very large catchments, all three metrics reach their maximum value of 1. 697 However, the skill curves do not increase continuously. Recall experiences a 698 loss in skill for areas ranging from 30,000 to 70,000 km², leading to a decrease 699 in f0.8 values. This loss is caused by missed events in the Guadiana, Seine, 700 Loire, Rhone and Danube catchments. As already mentioned in previous 701     sections, there is a notable gap between recall and precision, the components 702  of the f-score,  in both the optimized and current notification criteria,  with 703 precision consistently surpassing recall.

704	The vertical line at 2000 km² illustrates the gain in skill achieved through 705  optimization, which is sustained across all catchment area values. Reducing 706 the catchment area limit results in minimal loss in f0.8, suggesting that this 707 criterion could be reduced without compromising skill. For instance, the BW 708 approach at 1,000 km² exhibits slightly higher skill (f0.8 = 0.531) than the 709    current criteria at 2000 km² (f0.8 = 0.518). The relaxation of the catchment 710    area limit impacts the f-score components differently.   While precision re- 711    mains unaffected, there is a loss in recall. To sum up, issuing flood alerts to 712 smaller catchments would marginally reduce the skill of the system, primarily 713 due to an increase in missed events, without leading to more false alarms. This sets an opportunity to EFAS to increase its usability in even smaller river systems across Europe.

714	In the previous analysis, a fixed probability threshold was applied for all 715 catchments, regardless of their drainage area. We also investigated whether 716 the skill of the system could be enhanced by adapting the probability thresh- 717 old to the catchment area. Our rationale is rooted in the understanding 718 that the level of uncertainty (model spread) differs between small and large 719      catchments, suggesting that area-specific probability thresholds may enhance the 720 overall system skill. The results of this experiment are depicted in Figure 721    13. Each plot represents a different combination method, comparing the f- 722   score (solid lines) and the probability threshold (dashed lines) between the 723 model optimized for a 2,000 km² area limit (blue) and models optimized for 724    increasing catchment area limits (orange).  In order to compare the results 725    of the two optimization runs,  we kept a constant persistence value of 1/1, 726 which was identified as optimal in Table 4.

727	The skill measured as f0.8 is very similar for both optimizations, especially 728 for areas smaller than 2,000 km². For larger areas, the main improvement 729 is at the range between 30,000 and 100,000 km², for which the fixed criteria 730    loses some skill. The evolution of the probability threshold with catchment 731 area is erratic, likely due to the uncertainty in defining the optimal value 732 (refer to Figure 10). Despite this, a general trend can be discerned, with

1 deterministic + 1 probabilistic

persistence = 1/1

1.0

0.8

skill

0.6

model mean

member weighted

brier weighted

persistence = 1/1

1.0

0.8

0.6

f0.8 (optimal)

f0.8 (area optimized) prob. (optimal)

probability

prob. (area optimized)

persistence = 1/1

persistence = 1/1

0.4	0.4

0.2	0.2

0.0

103

104	105

area    (km²)

103

104	105

area    (km²)

103

104	105

area    (km²)

103

104	105

area    (km²)

0.0

Figure 13: Comparison of the notification skill between a fixed (blue) and a area-specific (orange) probability threshold criterion for the lead time range from 2 to 5.5 days. Each plot depicts a different combination method, with solid lines representing the f0.8 and dashed lines representing the probability threshold. The black, dotted, vertical line in- dicates the catchment area limit (2,000 km²) used for optimizing the fixed probability threshold.

733 the threshold taking lower values for catchments smaller than 2,000 km², 734   peaking at approximately 100,000 km²,  and dropping to a minimum of 5% 735 for very large catchments.

736	Overall, the experiment indicates that varying the probability threshold 737 does not significantly improve the skill of the notification criteria. Therefore, 738 for the sake of simplicity, a fixed probability threshold is recommended.

739    5.  Discussion

5.1. Limitations and moving forward

740	The definition of the spatial and temporal framework is often a limitation 741 in studies that analyze the skill of ensemble flood forecasting systems Cloke 742 and Pappenberger (2009). Due to the rarity of flood events, the statistical 743 robustness of such analysis relies on a limited and often highly correlated 744 sample of events.   To address this limitation, long-term and continental or 745 global studies are advocated, such as the one presented here, which spans 746 over a period of two and a half years and extends over a European domain. 747   Nevertheless, our study would benefit from a larger sample of points and flood 748 events. As shown in Figure 11, there are many stations for which the skill 749 metrics cannot be calculated due to null hits, misses or false alarms. A larger 750 sample of observed events would reduce the susceptibility of the analysis to 751    this issue. Two potential solutions emerge: extending the study period, which 752 is limited by the availability of the HEPS forecasts (or reforecasts), or using 753 an event threshold of higher recurrence. We decided to work with a sample 754    of stations, instead of all the river cells Bartholmes et al. (2009), to limit 755 the spatial correlation of events. Additionally, we tested removing stations

756 with highly correlated discharge time series to further reduce correlation, but 757 the results were similar and the sample of events much smaller; hence, this 758 approach was discarded.

759	In the optimization of the notification criteria, we set a catchment area 760 limit of 2,000 km², a threshold inherited from previous EFAS setups with 761   lower spatial and temporal resolution.   The results of this study (Figure 762    12) demonstrate that the current resolution of both NWP and hydrological 763 models can produce skillful warnings at smaller catchments, which can signif- 764 icantly increase the number of stations and flood events analyzed, as shown 765   in Figure 3. In line with this, the new EFAS version 5 (Joint Research Centre 766    - European Commission (2023)) has increased spatial resolution from 5 km 767 to 1 arc-minute, which is expected to further improve the skill of the system 768 in smaller catchments. The skill of this new EFAS version will be analyzed 769 as soon as a long enough period or reforecasts are available.

770	The selection of the target metric is a critical aspect in any skill analy-

771 sis. Bartholmes et al. (2009) reviewed the desired characteristics of a skill 772 score, and selected odds ratio, HK score and bias as their target metrics. 773    In contrast,  for our analysis,  we selected the fβ  score, a metric commonly 774   used in machine learning, but rarely applied in hydrology.  The fβ score has 775  two interesting properties for analyzing a flood warning system. Firstly, it is 776 based on the contingency table but avoids using true negatives, which could 777 overestimate skill. Secondly, it allows for tailoring the notification criteria to 778 the specific use case, as the β parameter can be adjusted to penalize stronger 779   misses or false alarms.  For instance, Bouttier and Marchal Bouttier and 780 Marchal (2023) used the f2 score to optimize the probability threshold for 781   high-intensity precipitation warnings in France.  The β  value of 2 penalizes 782 four times more missed events than false alarms, under the assumption that 783 the cost of being hit by an extreme event unprepared is larger than the cost 784    of mobilising resources in a false alarm. In our study, we took the opposite 785    approach and decided to minimize the number of false alarms by using a β 786 value of 0.8. This decision was based on the fact that EFAS complements 787 the national or regional systems by issuing medium-range flood warnings to 788 the responsible administrations, not to the general public.  The aim is to 789   alert authorities only when there is substantial evidence that the event will 790    occur,  in order to avoid excessive notifications that could undermine their 791  trust in the system. Additionally, the event threshold (5-year return period) 792 corresponds to a recurrence time shorter than the flood protection levels in 793   many European regions, so we assume that missing minor events will not be

794 as costly as Bouttier and Marchal argue. In a preliminary analysis, we tested 795 β values of 0.8, 1 and 1.25 and found that the changes in the f-score were 796     minimal (in the order of 3%) and in none of the cases persistence was needed. 797 However, the selection of β affects the optimal probability threshold and the 798    resulting bias, i.e., the distribution of errors between misses and false alarms. 799 Values lower than 1 lead to higher probability thresholds and negative biases, 800 whereas values larger than 1 yield positive bias, i.e., more notifications than 801 actual events.

5.2. Addressing study objectives

802	The first objective of this study was to evaluate the skill of the differ-

803  ent NWP within EFAS, with a specific focus on comparing deterministic and 804 probabilistic models. Figure 4 demonstrates that probabilistic NWPs outper- 805   form deterministic models in terms of Brier skill score across all lead times, 806 with the exception of short lead times, where deterministic NWPs approach 807    similar levels of skill. The outcome is similar when applying the notification 808   criteria and measuring warning skill in terms of f0.8  (Figure 5 and Figure 6). 809  At their  optimal notification  criteria,  the probabilistic  NWPs,  particularly 810 ENS, outperform deterministic NWPs.  The skill of both types of NWP de- 811 grades with increasing lead time, casting doubt on their ability to issue flood 812   alerts more than 5-6 days in advance. The effects of the probability thresh- 813 old and persistence criteria differs depending on the nature of the NWP. 814 The probability threshold has a substantial effect on the skill of probabilistic 815    models, while it does not impact deterministic models.  Both COS and ENS 816 perform better at probability thresholds over 50%, with a wide range of sim- 817 ilarly performing values. On the contrary, persistence proves to be beneficial 818 in improving the notification skill of deterministic NWPs, but it limits the 819   skill of probabilistic models.  Persistence,  a.k.a.  poor man’s ensemble,  is a 820 method of creating an ensemble out of a deterministic forecast Cloke and 821  Pappenberger (2009). For instance, a persistence of 3/3 is an ensemble of 3 822  members with a probability threshold of 100%. Our results show that while 823 persistence removes the inherent erratic behaviour of deterministic models 824 and notably improves their skill, it should not be applied to HEPS.

825	The second objective of this study was to determine the appropriate

826 method to combine several NWP models into a grand ensemble. Careful 827  consideration is necessary in selecting this method to avoid diminishing the 828 skill of the most proficient NWP within the grand ensemble. For instance, 829 the current EFAS notification criteria limit the skill of the system to that 830 of the highest-performing deterministic model (HRES), which is poorer than 831     any of the probabilistic NWP (Figure 6). This limitation is caused by a com-

832   bination method (1D+1P) that excessively relies on deterministic forecasts, 833 as well as the use of persistence. The challenge lies in finding a combination 834 that enhances the skill of the best-performing NWP, which in the case of 835 EFAS is the ENS.

836	Our findings demonstrate that ENS is a very strong baseline, with only a 837  few combination methods yielding gains to its skill. The effectiveness of the 838 grand ensemble is limited to the first 5 days lead time, when the total number 839    of members is highest (73), particularly for lead times shorter than 2 days, 840 when the deterministic models provide value. As an example, a simplistic 841 approach like MM exhibits the highest skill within the shortest lead time 842 range, potentially because it is the approach that assigns higher value to 843   the deterministic models.  Overall,  the most promising approaches are the 844 member-based (MW) and skill-based (BW) methods, as they outperform the 845    baseline throughout the first 5 days lead time.  Both methods have similar 846 optimal notification criteria (no persistence and a probability threshold close 847 to 50%), and their success is based on granting most of the weight to ENS. 848 However, the reason why they allocate this weight to ENS differs. In the 849 case of MW, it is coincidental that the most skillful model also possesses 850    the largest number of members.  On the contrary, BW grants greater value 851 to ENS because it proved to be a superior model. If a more skillful model 852  with fewer members would be added to the grand ensemble, BW would be 853    able to extract that value, while MW would not.  Therefore, we argue that 854 skill-based combination methods are preferable.

5.3. Towards operationalization

855	The decision on the most appropriate combination methods can not be

856    based solely on skill metrics, but also consider other factors such as the opera- 857 tional implementation effort, user comprehensibility, or the impact of adding 858 or removing a NWP. Skill-based approaches, such as BW, are theoretically 859 superior as the allocation of weights is based on quality rather than quantity. 860 They are also the most robust when it comes to adding or removing NWP 861 models. However, they are more challenging to implement and to explain to 862 end users. To derive the matrix of weights (Figure 8), a skill-based approach 863 requires having an extensive period of reanalysis and reforecasts upfront, 864   posing a substantial computational burden on system implementation. Ad- 865 ditionally, practitioners using the forecasts would require additional training 866   to understand that the ensemble members are not equiprobable, which would 867 be more intuitive, but there are some prevailing members.

868	There remains a question on the use of deterministic models in a grand

869    ensemble dominated by a probabilistic NWP. When using a member-based

870 combination, such as MW, deterministic models could be removed from the 871 grand ensemble, as the weight allocated to them is so small that their in- 872   clusion barely affects the total probability. Skill-based approaches, instead, 873 allocate larger weights to these models in the very first lead times, when 874  they have proven value given their often higher resolution. The inclusion of 875 deterministic models is a topic for future analysis, as the spatial resolution of 876 deterministic and probabilistic NWP are nowadays similar, and centres such 877 as the ECMWF plan to discontinue their deterministic models in favour of 878 the ensembles.

879    6.  Conclusions

880	This study describes a procedure to optimize warning thresholds in HEPS. 881 We investigated the ability of deterministic and probabilistic NWP mod- 882    els in providing correct flood warnings,  and we explored the effects of the 883    notification criteria,  namely probability threshold and persistence.  The fi- 884 nal objective of the study is to device a method to combine several NWP 885 into a grand ensemble in order to outperform the most proficient, individual 886 NWP. Our study case is the European Flood Awareness System (EFAS v.4), 887 a medium-range continental flood warning system.

888	The  individual  analysis  of  the  NWPs  demonstrated  that  probabilistic

889    models, particularly ENS, provide better flood warnings than deterministic 890   models, even for short lead times, where the latter show some skill.   We 891 discovered that the persistence criterion is useful to increase the skill of de- 892      terministic models, as it creates a pseudo-ensemble, but is counterproductive 893 in the case of ensembles. The criterion that maximizes ensemble skill is the 894 probability threshold, whose optimum shows a certain degree of uncertainty, 895 but always exceeds a value of 50%.

896	By default, the combination of several NWP into a grand ensemble does

897 not improve the overall skill of the flood warning system. As an example, the 898 current EFAS notification criteria, which uses four NWPs, is outperformed 899 by only using one NWP (ENS) with the adequate criteria.  Two of the ap- 900  proaches tested in this work yielded gains in skill compared with the baseline 901  in medium and short lead times. One of these approaches (named member 902 weighted) assumes equiprobability among all the model runs, and the other 903 one (named Brier weighted) assigns weights to each NWP at every lead time 904 based on their probabilistic skill. In our study case, these two approaches 905    yielded similar results because the most proficient NWP is also the one with

906 the largest number of members. Despite that, we argue that the skill-based 907     approach is conceptually more appropriate, gives significance to the inclusion 908 of deterministic NWP in the grand ensemble, and would perform better if 909   the set of NWP would change.   However, other study cases may consider 910 other factors such implementation costs or explainability in the selection of 911 the combination method.

912	Finally, we explored how catchment area affects the notification skill, 913 with the idea of relaxing the current notification criterion that establishes a 914   minimum catchment area.   The results indicate that skill deteriorates with 915 decreasing catchment area, as other studies report, but the improvements 916 in meteorological and hydrological models over the last decade allow for a 917 reduction in the minimum catchment area. We have proven that with the 918 optimized criteria we can halve the area limit, therefore cover a larger area, 919 and achieve a similar performance as in the current system setup. We expect 920 that the skill at smaller catchments will be positively affected by the release 921 of the new EFAS version with a higher spatial resolution.

# 922  7. Data availability

923	The reanalysis and forecast discharge data is available at the Climate 924 Data Store of the Copernicus Climate Change Service.  All the scripts, pre- 925    processed data and results of the study can be found in this GitHub reposi- 926 tory.

927     References

928 Alfieri, L., Bisselink, B., Dottori, F., Naumann, G., de Roo, A., Salamon, 929	P., Wyser, K., Feyen, L., 2017. Global projections of river flood risk in a 930	warmer world. Earth’s Future 5, 171–182. doi:10.1002/2016EF000485.

931 Bartholmes, J.C., Thielen, J., Ramos, M.H., Gentilini, S., 2009. The eu- 932	ropean flood alert system efas – part 2: Statistical skill assessment of 933	probabilistic and deterministic operational forecasts. Hydrol. Earth Syst. 934	Sci 13, 141–153. URL: www.hydrol-earth-syst-sci.net/13/141/2009/,

935	doi:10.5194/hess-13-141-2009.

936     Bauer, P., Thorpe, A., Brunet, G., 2015.  The quiet revolution of numerical

937	weather prediction. Nature 525, 47–55. doi:10.1038/nature14956.

938 Bouttier,  F.,  Marchal,  H.,  2023.  Probabilistic short-range forecasts of high 939	precipitation events : optimal decision thresholds and predictability limits. 940	URL:      https://doi.org/10.5194/egusphere-2023-3111,      doi:10.5194/ 941	egusphere-2023-3111.

942     Brier, G.W., 1950. Verification of forecasts expressed in terms of probability.

943	Monthly Weather Review 78.

944     Buizza, R., 2008. The value of probabilistic prediction. Atmospheric Science

945	Letters 9, 36–42.  doi:10.1002/asl.170.

946     Burek, P., van der Knijff, J., de Roo, A., 2013. Lisflood. distributed water

947	balance and flood simulation model. doi:10.2788/24719.

948     Cloke, H.L., Pappenberger, F., 2009. Ensemble flood forecasting: A review.

949	Journal    of    Hydrology    375,    613–626.       doi:10.1016/j.jhydrol.2009.06.

950	005.

951   Demeritt, D., Nobert, S., Cloke, H.L., Pappenberger, F., 2013. The european 952	flood alert system and the communication, perception, and use of ensemble 953	predictions for operational flood risk management. Hydrological Processes 954	27, 147–157. doi:10.1002/hyp.9419.

955 Diomede, T., Davolio, S., Marsigli, C., Miglietta, M.M., Moscatello, A., Pa- 956	petti, P., Paccagnella, T., Buzzi, A., Malguzzi, P., 2008.  Discharge pre- 957	diction based on multi-model precipitation forecasts.   Meteorology and 958	Atmospheric Physics 101, 245–265. doi:10.1007/s00703-007-0285-0.

959   EEA,  2023.   Economic  losses  from  weather-  and  climate-related  ex- 960	tremes in Europe. Technical Report. European Environmental Agency. 961	URL:	https://www.eea.europa.eu/en/analysis/indicators/

962	economic-losses-from-climate-related?activeAccordion=

963	ecdb3bcf-bbe9-4978-b5cf-0b136399d9f8.

964 Egerton, P., Stander, J., Paterson, L., Devillier, R., Champagne-Fall, A., 965	Honore, C., Allis, E., Love, G., Nitu, R., Stuart, L., Kaya, F., Santamaria, 966	L., Teruggi, G., Taalas, P., Rea, A., Lucio, F., Harding, J., Repnik, M., 967	Luterbacher, J., Alexieva, A., Gonsior, H., Uhlenbrook, S., Cullmann, J., 968	Lombardi, N., Dunja, D., Pfister, D., Zommers, Z., Escobar Rodriguez, 969	H., Jacob, A., Keo, K., 2023. Early warnings for all. Executive action plan

970	2023-2027.  Technical Report. Unitated Nations.  URL: https://www.un.

971	org/en/climatechange/early-warnings-for-all.

972 Gupta,  H.V.,  Kling,  H.,  Yilmaz,  K.K.,  Martinez,  G.F.,  2009.   Decompo- 973	sition of the mean squared error and nse performance criteria:  Implica- 974	tions  for improving  hydrological  modelling.   Journal of Hydrology 377, 975	80–91. URL: https://www.sciencedirect.com/science/article/pii/

976	S0022169409004843,   doi:10.1016/J.JHYDROL.2009.08.003.

977    Hanssen, A., Kuipers, W., 1965. On the Relationship Between the Frequency 978	of Rain and Various Meteorological Parameters: (with Reference to the 979	Problem Ob Objective Forecasting).    Koninkl. Nederlands Meterologisch 980	Institut. Mededelingen en Verhandelingen, Staatsdrukkerij- en Uitgeveri- 981	jbedrijf.

982    Joint Research Centre - European Commission,  2020.   European Flood 983	Awareness    System    v4.0.       URL:    https://confluence.ecmwf.int/ 984	display/CEMS/EFAS+v4.0.

985   Joint  Research  Centre  -  European  Commission,  2023.    European  Flood 986	Awareness     System     v5.0.          URL:     https://confluence.ecmwf.int/ 987	display/CEMS/Latest+operational+EFAS+release.

988  Kling,  H.,  Fuchs,  M.,  Paulin,  M.,  2012.   Runoff  conditions  in  the  upper 989	Danube basin under an ensemble of climate change scenarios. Journal of 990	Hydrology 424-425, 264–277. doi:10.1016/j.jhydrol.2012.01.011.

991   Knoben,  W.J.,  Freer,  J.E.,  Woods,  R.A.,  2019.   Technical  note:  Inherent 992	benchmark or not? comparing nash-sutcliffe and kling-gupta efficiency 993	scores. Hydrology and Earth System Sciences 23, 4323–4331.  doi:10. 994	5194/hess-23-4323-2019.

995     LeClerc, J., Joslyn, S., 2015. The cry wolf effect and weather-related decision

996	making.  Risk Analysis 35, 385–395.  doi:10.1111/risa.12336.

997     Legg,  T.P.,  Mylne,  K.R.,  2004.	Early Warnings of Severe Weather from

998	Ensemble Forecast Information. Weather and Forecasting 19, 891–906.

999

1000

1001

Martina   Angela   Caretta,   Aditi   Mukherji,   Md   Arfanuzzaman,   Richard

  - Betts, Alexander Gelfan, Yukiko Hirabayashi, Tabea Katharina Lissner,	Elena   Lopez   Gunn,	Junguo    Liu,	Ruth   Morgan,	Sixbert

1002

Mwanga,	Seree	Supratid,	2023.	Water,	in:	Climate	Change

| 1003 | 2022 – Impacts, Adaptation and Vulnerability. Cambridge University |
| --- | --- |
| 1004 | Press, pp. 551–712. URL: https://www.cambridge.org/core/product/ |
| 1005 | identifier/9781009325844{%}23c4/type/book{_}part, doi:10.1017/ |
| 1006 | 9781009325844.006. |
| 1007 | Molteni, F., Buizza, R., Palmer, T.N., Petroliagis, T., 1996. The ECMWF |
| 1008 | Ensemble Prediction System: Methodology and validation. Quarterly |
| 1009 | Journal of the Royal Meteorological Society 122, 73–119. doi:https: |
| 1010 | //doi.org/10.1002/qj.49712252905. |
| 1011 | Nevo, S., Morin, E., Gerzi Rosenthal, A., Metzger, A., Barshai, C., Weitzner, |
| 1012 | D., Voloshin, D., Kratzert, F., Elidan, G., Dror, G., Begelman, G., Near- |
| 1013 | ing, G., Shalev, G., Noga, H., Shavitt, I., Yuklea, L., Royz, M., Giladi, |
| 1014 | N., Peled Levi, N., Reich, O., Gilon, O., Maor, R., Timnat, S., Shechter, |
| 1015 | T., Anisimov, V., Gigi, Y., Levin, Y., Moshe, Z., Ben-Haim, Z., Hassidim, |
| 1016 | A., Matias, Y., 2022. Flood forecasting with machine learning models |
| 1017 | in an operational framework. Hydrology and Earth System Sciences 26, |
| 1018 | 4013–4032. doi:10.5194/hess-26-4013-2022, arXiv:2111.02780. |
| 1019 | Pappenberger, F., Cloke, H.L., Parker, D.J., Wetterhall, F., Richardson, |
| 1020 | D.S., Thielen, J., 2015. The monetary benefit of early flood warnings in |
| 1021 | Europe. Environmental Science and Policy 51, 278–291. doi:10.1016/j. |
| 1022 | envsci.2015.04.016. |
| 1023 | Paprotny, D., Sebastian, A., Morales-N´apoles, O., Jonkman, S.N., 2018. |
| 1024 | Trends in flood losses in Europe over the past 150 years. Nature Com- |
| 1025 | munications 9. doi:10.1038/s41467-018-04253-1. |
| 1026 | Roebber, P.J., 2009. Visualizing multiple measures of forecast quality. |
| 1027 | Weather and Forecasting 24, 601–608. doi:10.1175/2008WAF2222159.1. |
| 1028 | Thielen, J., Bartholmes, J., Ramos, M.H., Roo, A.D., 2009. The european |
| 1029 | flood alert system – part 1: Concept and development. Hydrol. Earth Syst. |
| 1030 | Sci 13, 125–140. URL: www.hydrol-earth-syst-sci.net/13/125/2009/, |
| 1031 | doi:10.5194/hess-13-125-2009. |
| 1032 | WMO, 2021. WMO Atlas of mortality and economic losses from weather, |
| 1033 | climate and water extremes (1970-2019). Technical Report. World Meteo- |
| 1034 | rological Organization. Geneva. |

1035

1036

1037

1038

Yang, S.C., Yang, T.H., Chang, Y.C., Chen, C.H., Lin, M.Y., Ho, J.Y., Lee,

K.T., 2020.   Development of a hydrological ensemble prediction system to assist with decision-making for floods during typhoons. Sustainability (Switzerland) 12. doi:10.3390/su12104258.